Appendix 1 – Particular Conditions and Technical Specifications

Tender for

Supply and Deployment of Video Broadcasting Equipment

for the High Qualification Commission of Judges of Ukraine

Section A:	Particular Conditions

Section B:	Technical Specifications

- The Buyer’s Requirements

- The Supplier’s Technical Specifications

All sections which the Tenderer must complete are marked in yellow.

## Section A: Particular Conditions

### General background

The High Qualification Commission of Judges of Ukraine (HQCJ) is one of the program’s recipients, as it is responsible for conducting competitions for vacant positions at the High Anti-Corruption Court (HACC), one of the core EUACI beneficiaries. Consequently, the EUACI supports strengthening the HQCJ’s institutional capacity in areas important to ensure an efficient, transparent, and high-quality HACC competition.

To advance this objective, the EUACI intends to enhance the HQCJ’s capacity for public communication by improving the quality and accessibility of its livestreamed sessions, particularly during judicial selection processes. For this purpose, the EUACI will support the acquisition of the professional video-broadcasting equipment for the HQCJ. The upgraded system will enable high-quality livestreaming, reinforce the transparency and credibility of selection procedures, and reduce future rental and setup costs for donor-supported activities.

### Recipient

Under this Agreement, the Recipient of the Goods is not the Buyer but:

Name of Recipient: High Qualifications Commission of Judges of Ukraine

Name of representative: Valeriy Korobka

Postal address: Ukraine, Kyiv, 9 Henerala Shapovala 17, 03110

Email: korobkavv@vkksu.gov.ua

### Place of delivery

The Supplier shall deliver the Goods to:

Name of location: High Qualifications Commission of Judges of Ukraine

Name of representative: Valeriy Korobka

Postal address: Ukraine, Kyiv, 9 Henerala Shapovala 17, 03110

Office hours: 8:00-17:00

### Time of delivery

The Supplier shall deliver the equipment and perform the scope of work no later than:

Date: 6 February 2026

Time: 17.00 Kyiv time

Section B – Technical Specifications

The Supplier is obliged to deliver the Goods in accordance with the Buyer’s requirements and the Supplier’s Technical Specifications. In case of conflict between the Buyer’s requirements and the Supplier’s Technical Specifications, the Buyer’s Requirements shall prevail.

### The Buyer’s Requirements

  - Technical Specification of video broadcasting equipment:

PTZ Camera:

| Category | Specification |
| --- | --- |
| Camera Type | PTZ camera with 30x optical zoom, 4K resolution |
| Video Outputs / Protocols | HD-SDI, HDMI, IP (H.264) |
| Aspect Ratio | HD-SDI: 16:9 (all resolutions) HDMI: 16:9 (all resolutions) IP Streaming (H.264): 16:9, 3:2, 4:3 |
| Video Resolutions / Frame Rates | FHD: 1080p/60/59.94/50/29.97/25; 1080i/60/59.94/50 HD: 720p/60/59.94/50 |
| Streaming Resolutions / Frame Rates | IP (RTSP/RTMP, H.264) up to 1080p/30 (simultaneous) |
| Image Sensor | 1/2.5” Exmor R CMOS |
| Effective Pixels | 8.51 MP (8.57 MP total) |
| Lens / Zoom | 30E Model: 30x zoom (20x optical + 10x IntelliZoom), f = 4.4mm–88.4mm, F2–F3.8 |
| Horizontal Field of View | Not specified (dependent on lens range) |
| Pan Range / Speed | ±150°, 0.35°/sec to 120°/sec |
| Tilt Range / Speed | +90° / -30°, 0.35°/sec to 120°/sec |
| Minimum Working Distance | 80 mm (wide), 800 mm (tele) |
| Minimum Illumination | 1.6 lux; 100+ lux recommended |
| Electronic Shutter Speed | 1/1 to 1/10,000 sec |
| Backlight Compensation | On / Off |
| Aperture / Detail | 16 steps |
| Focus Modes | Auto / Manual / One-push Trigger |
| Gain | Auto / Manual (0–33 dB) |
| White Balance | Auto, ATW, Indoor, Outdoor, One-push, Manual |
| Power | PoE+ |
| Remote Control Interfaces | IR remote Web interface Telnet, RS-232 APIs |
| Image Enhancements | Enhanced WDR, Spotlight Compensation, Noise Reduction |
| Pre-set Positions | 16 + Custom Home |
| Invertible Mounting | Yes |
| Motor Type | Silent BLDC Direct Drive Motors |
| Warranty | 2 years (optional +3 years extension) |
| Weight | 4.85 lbs (2.2 kg) |
| Dimensions (H × W × D) | 6.94" × 7.05" × 6.75" (17.63 × 17.91 × 17.15 cm) |
| Operating Temperature | 0°C to +40°C (32°F to 104°F) |
| Operating Humidity | 20%–80% RH, non-condensing |

HDMI Switch with Integrated Multi-View & Video Capture:

| Category | Specifications |
| --- | --- |
| Video Processing | • 4×2 HDMI multi-viewer mode • Seamless switching (no loss of sync) • HDMI I/O full 4K 6G compatible with auto conversion for USB capture • Video timing stabilization for reliable USB capture • Supports Custom Size, Position, and PIP Window Mode |
| USB Capture | • USB 3.0 capture of uncompressed video/audio (1080p, LPCM 2.0) • Compliance: UAC (USB Audio Class), UVC (USB Video Class) • Captures 2-channel digital audio from HDMI or 3.5mm analogue input (selectable) |
| Control & Management | • Front panel buttons • Web GUI • Telnet • RS-232 • Dedicated video capture software |
| Inputs | • 4× HDMI (uncompressed AV & data) • 1× HDMI capture (uncompressed AV & data) • 1× USB 3.0 Type B (host PC power & data) • 1× 3.5mm stereo mini-jack (analog L/R) • 1× RJ45 RS-232 (proprietary data) • 1× RJ45 LAN control & stream out (proprietary data) |
| Outputs | • 3× HDMI (uncompressed AV & data) |
| Power Supply | • 24V DC, 2.7A |
| Physical | • Dimensions: 231.5 mm [W] × 43 mm [H] × 158 mm [D] • Net Weight: 1.18 kg • Gross Weight: 2.32 kg |

Camera control processor:

| Category | Specifications |
| --- | --- |
| Memory / Storage | • SDRAM: 512 MB • Flash: 4.5 GB |
| Software & Configuration | • Configuration: Global Configurator Plus & Professional • Programming: Global Scripter • Control Apps: Extron Control • Management: GlobalViewer Enterprise • Utilities: Toolbelt, embedded web page |
| Compatible Hardware | TouchLink Pro touchpanels, NBP button panels, eBUS button panels |
| Network Interfaces | • NICs: 2 (1 LAN, 1 AV LAN) • Connectors: 2× RJ-45• Data rate: 10/100/1000Base-T, half/full duplex, autodetect • Protocols: DHCP, DNS, HTTP (redirect), HTTPS, ICMP, IEEE 802.1X, NTP, SFTP, SMTP, SNMP, SSH, TCP/IP, UDP/IP |
| Default Network Settings | LAN: - IP: 192.168.253.250 - Subnet: 255.255.255.0 - Gateway: 0.0.0.0 - DHCP: off— DNS: 127.0.0.1 AV LAN: - DHCP server: disabled - IP: 192.168.254.250 - DHCP server enabled: IP: 192.168.254.1 DNS: 192.168.254.1 Leased pool: 192.168.254.100–149 Max leases: 50Lease time: 24 hrs |
| Serial Ports | • Qty/Type: 2— Port 1: RS-232/422/485 bidirectional— Port 2: RS-232 bidirectional • Connectors: 5-pole and 3-pole captive screw • Baud rate: 300–115200 (9600 default), 7/8 data bits, 1/2 stop bits, parity: none/even/odd |
| Serial Pinouts | 5-pole: RS-232: Tx, Rx, Gnd, RTS, CTSRS-422: Tx–, Rx–, Gnd, Tx+, Rx+RS-485: data–, Gnd, data+ 3-pole: Tx, Rx, Gnd |
| Digital I/O | • Qty: 4 configurable I/O • Connector: 5-pole captive screw • Input voltage: 0–24 VDC (clamped at +30 VDC) • Input impedance: >10kΩ• Pull-up: 1kΩ to +5 VDC • Threshold: 2.8 V high, 2.0 V low • Output: 250 mA sink @ 24 VDC max |
| IR / RS-232 Programmable Port | • Qty: 1 • Type: RS-232 or IR (30–300 kHz) • Connector: 2-pole captive screw • IR learning distance: 2" to 12" (5–30 cm) |
| Relays | • Qty: 2 normally open • Connector: 3-pole captive screw • Rating: 24 VDC, 1 A |
| Volume Control Port | • Qty: 1 • Connector: 5-pole captive screw • Control voltage: 0–10 VDC (±0.2 V) • Ref input: ≤10 VDC |
| eBUS Port | • Qty: 1 • Connector: 5-pole captive screw (4 poles used) • Power output: 6 W • Max cable length: 1000 ft (305 m) • Recommended cable: Extron STP20-2/1000 |
| Power | • External power supply included— Input: 100–240 VAC, 50–60 Hz— Output: 12 VDC, 1.5 A (18 W) • Device power requirement: 12 VDC, 1 A (12 W max) • Device consumption: 9.0 W (3.5 W without eBUS) • Device + PSU: up to 11.9 W |
| Environmental | Operating: +32 to +122 °F (0 to +50 °C), 10–90% RH (non-condensing) Storage: –40 to +158 °F (–40 to +70 °C), 10–90% RH |
| Cooling | Convection, no vents |
| Thermal Dissipation | • Device: 10.1 BTU/hr • Device + PSU: 20.1 BTU/hr |
| Mechanical | • Mounting: Rack-mount (1U, optional shelf), furniture mount • Enclosure: Metal • Dimensions: 1.7" H × 4.3" W × 6.0" D (43 × 109 × 152 mm) • Weight: 0.7 lbs (0.3 kg) |
| Regulatory Compliance | CE, C-Tick, c-UL, FCC Class A, ICES, KC, UL, plenum-rated |

Tablet for camera control:

| Category | Specifications |
| --- | --- |
| Display | • Type: TFT active-matrix color LCD • Size: 7" (17.8 cm) diagonal • Resolution: 1024×600 • Pixel pitch: 170 • Aspect ratio: Widescreen • Color depth: 24-bit (16.7M colors) • Transparency: 8-bit • Brightness: 420 nits • Contrast: 700:1 • Backlight: LED • Viewing angle: ±75° horizontal, +70°/–75° vertical • Touch technology: Capacitive |
| Memory / Storage | • SDRAM: 2 GB • Flash: 8 GB |
| Network & Communication | • Ethernet port: 1× RJ-45 • Data rate: 10/100/1000Base-T, half/full duplex, autodetect • Supported protocols: DHCP, DNS, HTTP, HTTPS, ICMP, IEEE 802.1x, SFTP, SSH, TCP/IP, UDP/IP • Default settings: — IP: 192.168.254.251— Subnet mask: 255.255.255.0— Gateway: 0.0.0.0— DHCP: Off— DNS: 127.0.0.1 |
| Software & Configuration | • Configuration: Global Configurator Plus/Professional, GUI Designer • Programming: ControlScript • Control apps: Extron Control • Management: GlobalViewer Enterprise • Utilities: Toolbelt, embedded web interface |
| Sensors | • Motion sensor: On/Off • Light sensor: On/Off |
| USB Interface | • Ports: 1× USB 2.0 Type-A (device) • Standards: USB 2.0, 1.1, 1.0 • Speeds: 1.5 Mbps (low), 12 Mbps (full) |
| Audio Playback | • Speaker: 1× mono, 86 dB SPL, 0.1 W at 0.1 m (half-space) • Frequency response: 750 Hz–20 kHz, ±5 dB • Supported format: WAV (Microsoft RIFF) • Encoding: LPCM (uncompressed) • Bit depth: 8-bit unsigned; 16/24/32-bit signed • Sample rate: up to 192 kHz • Channels: Mono or stereo |
| Power | • Power input: Power over Ethernet (PoE 802.3af, Class 3) • Power consumption (device): 8.2 W • Power consumption (with injector): 10.6 W |
| Environmental | • Operating temperature: –4 to +158 °F (–20 to +70 °C) • Storage temperature: –4 to +158 °F (–20 to +70 °C) • Operating humidity: 20–90% (non-condensing) • Storage humidity: 5–90% (non-condensing) |
| Mechanical | • Cooling: Convection (rear vents) • Mounting: Tabletop • Enclosure: Plastic • Dimensions: 4.78" H × 7.12" W × 3.53" D (122 × 181 × 90 mm) • Weight: 1.75 lbs (0.79 kg) |
| Regulatory Compliance | BIS, CE, C-Tick, c-UL, FCC Class B, ICES, RoHS, UL, VCCI, WEEE |

HDMI twisted pair signal receiver and transmitter set:

| Category | Specifications |
| --- | --- |
| Technology | HDBaseT extension |
| Video Support | • Uncompressed HDMI video • Full HD 1080p @ 60 Hz • 3D support • 4K×2K support |
| Transmission | • Extend 1080p signal via Cat6a up to 100 meters • Bidirectional IR pass-through |
| Standards & Compatibility | • HDCP, CEC • 24-bit deep color support |
| Audio Support | • Uncompressed LPCM audio • Compressed formats: DTS-HD, Dolby TrueHD |

8-port POE switch:

| Category | Specifications |
| --- | --- |
| Interface | 8× RJ45 ports, 10/100/1000 Mbps |
| Auto-sensing / MDI/MDIX | Yes, automatic detection |
| Network Environment / Cable Support | • 10BASE-T: UTP Cat 3, 4, 5 (max 100 m), STP 100 Ω (max 100 m) • 100BASE-TX: UTP Cat 5/5e (max 100 m), STP 100 Ω (max 100 m) • 1000BASE-T: UTP Cat 5/5e/6 or higher (max 100 m), STP 100 Ω (max 100 m) |
| Fans | Fanless |
| Physical Security | Yes |
| Power Consumption | • Max (PoE on): 63 W (220 V / 50 Hz) • Max (PoE off): 3.3 W (220 V / 50 Hz) |
| External Power Supply | External adapter: Output 53.5 VDC / 1.31 A |
| Packet Forwarding Rate | 11.904 Mpps |
| PoE Ports | • Standard: IEEE 802.3af compliant • Ports: 1–4 • Power budget: 53 W |
| Backplane Bandwidth | 16 Gbps |
| MAC Address Table | 4,000 entries |
| Jumbo Frame Support | 16 KB |
| Switching Capacity / Throughput | 16 Gbps |
| Dimensions (W × D × H) | 171 × 98 × 27 mm |

### The Supplier’s Technical Specifications

The Supplier must provide a table of correspondence of the proposed solution with the manufacturer and model to the technical specification.

[insert reference to attachment and/ or insert a description]

Signature

I, the undersigned, hereby declare that:

- I am empowered to represent the tenderer with mandate to establish a legal obligation on behalf of the tenderer in relation to the Buyer.

- I have familiarised myself with the information available to date concerning this tender procedure.

- The submitted tender is fully compliant with all requirements and that all statements, descriptions, etc. provided by the Seller in this Appendix 1 are accurately and correct.

- The statements in this Appendix 1, including attached Technical Specifications, take precedence over the Supplier’s other statements and/ or indications in the tender, if any.

- The submitted tender will be deemed to be the property of the Buyer.

| Name | [insert name of the undersigned] |
| --- | --- |
| Position | [insert name of the undersigned] |
| Date | [insert date] |
| Signature |  |